2.5G Base SMD Transformer

Part Number: SHG24N13P5H4

| ● Compliant with IEEE 802.3 standard. ● RoHS compliant. ● Power over Ethernet(PoE). With 1A current for two pair ● Operating temperature range: -0°C to +75°C. ● Storage temperature range: -40°C to +125°C |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Electrical Specifications @ 25°C |  |  |  |  |  |  |  |  |  |  |
| Part Number | Turns Ratio (±2%) |  |  | Insertion Loss (dB Max) |  |  |  |  |  | HI-POT (Vrms) |
|  | TX | RX |  | 1-100MHz |  |  |  | 100-125MHz |  |  |
| SHG24N13P5H4 | 1CT:1CT | 1CT:1CT |  | 1.0 |  |  |  | -1.2 |  | 4000 |
| Continue |  |  |  |  |  |  |  |  |  |  |
| Part Number | Return Loss (dB Min) |  |  |  | Cross Talk (dB Min) |  |  |  | DCMRR (dB Min) |  |
|  | 1-40 MHz |  | 40-125 MHz |  | 1 -60MHz |  | 60-100MHz |  | 1-30 MHz | 100-200 MHz |
| SHG24N13P5H4 | -16 |  | -16+10*log(f/40) |  | -37 |  | -30 |  | -40 | -28 |
| Schematics: |  |  |  |  |  | Mechanicals: |  |  |  |  |
|  |  |  |  |  |  | SHG24N13P5H4 Date Code ¡À |  |  |  |  |

R0(09/03)